# A respiratory protection program for employees required to wear respirators during normal work operations and emergencies

# Contents

Purpose	3

Scope and application	4

Responsibilities	5

Program administrator	5

Supervisors	5

Employees	6

Hazard evaluations	7

NIOSH certifications	8

Medical evaluations	9

Fit testing	11

Respirator use	12

General use	12

Voluntary use	12

Escape respirator	12

Respirator malfunction	13

Locations immediately dangerous to life or health	13

Air quality in atmosphere-supplying respirators	14

Cleaning, maintenance, change schedules, and storage	15

Cleaning	15

Maintenance	15

Change Schedules	16

Respirator storage	16

Training	18

Program evaluation	19

Documentation and recordkeeping	20

Appendix D to §1910.134 – Information for employees using respirators 
when not required under the standard (mandatory)	21

Fill-in forms	22

Form 1: Required respirators — determined through hazard assessments	22

Form 2: Respirators for voluntary use — determined through hazard assessments	23

Form 3: Work areas that require escape respirators	24

Form 4: Locations immediately dangerous to life or health	25

Form 5: Medical release	26

Form 6: Medical evaluation questionnaire from 1910.134, Appendix C	27

Form 7: Fit test results	34

Form 8: Program evaluation summary	35

# Purpose

This program ensures that [name of your company] employees are protected from airborne chemical hazards during their work. Engineering controls such as ventilation and substitution of less toxic materials are preferred protection methods. However, for some tasks and during emergencies respirators are necessary to protect employees. To ensure that employees who wear respirators are protected from airborne chemical hazards, [name of your company] will do the following:

- Evaluate respiratory hazards to ensure employees have appropriate respirators.

- Ensure that employees are medically able to wear respirators.

- Fit-test employees with the appropriate respirators.

- Train employees to use and maintain their respirators.

- Evaluate this program periodically to ensure that it is effective.

# Scope and application

This program applies to all employees who are required to wear respirators during their work and during emergencies such as spills of a hazardous substance. Employees participate in the respiratory protection program at no cost; the costs for medical evaluations, fit testing, and respirators will be paid by [name of your company].

# Responsibilities

## Program administrator

The program administrator is responsible for administering the respiratory protection program and has the following duties:

- Identify the work areas, processes or tasks that require employees to wear respirators and evaluate the hazards.

- Select appropriate respirators for employees.

- Ensure that employees use respirators in accordance with NIOSH certifications.

- Ensure that employees receive respiratory protection training.

- Ensure that employees store and maintain respirators properly.

- Manage respirator fit testing.

- Manage medical surveillance of employees.

- Maintain required records.

- Inform contractors of this company’s respiratory protection requirements.

- Evaluate the respiratory protection program.

- Update the respiratory protection program when necessary.

The program administrator is [name of the program administrator].

## Supervisors

Supervisors will ensure that the respiratory protection program is implemented in their work areas. Supervisors must understand the requirements of this program and ensure that the employees under their charge understand the requirements. Supervisors have the following responsibilities:

- Ensure that employees under their supervision have received appropriate training, fit testing, and medical evaluations.

- Ensure that appropriate respirators and accessories are available.

- Know the tasks that require respiratory protection.

- Enforce the proper use of respirators.

- Ensure that respirators are cleaned, maintained, and stored as required by this program.

- Monitor work areas to identify respiratory hazards.

- Work with the program administrator to address respiratory hazards and other program concerns.

## Employees

Employees must wear their respirators in the manner in which they were trained and do the following:

- Care for and maintain their respirators as instructed and store them in a clean sanitary location.

- Inform their supervisor if the respirator no longer fits and request a new one that fits properly.

- Inform their supervisor or the program administrator about respiratory hazards or other concerns that they have regarding the respiratory protection program.

# Hazard evaluations

The program administrator will select respirators based on the hazards to which workers are exposed and in accordance with Oregon OSHA requirements. The program administrator will conduct a hazard evaluation for each work process or area where airborne contaminants may be present during routine operations or emergencies. The evaluation must include the following:

- Identification and development of a list of hazardous substances used in the workplace by department or work process.

- Review of work processes to determine where potential exposures to these hazardous substances may occur. This review will be conducted by surveying the workplace, reviewing process records, and talking to employees and supervisors.

Hazard evaluations should also include exposure monitoring to quantify potential hazardous exposures.

The program administrator must revise and update the hazard assessment any time there are changes in the workplace that may affect exposure. Employees who feel that respiratory protection is necessary must contact their supervisor or the program administrator. The program administrator will evaluate the hazards and inform the employees about the evaluation results. If respiratory protection is necessary all elements of this program will apply and this program will be updated.

# NIOSH certifications

All respirators must be certified by the National Institute for Occupational Safety and Health (NIOSH) and used according to the terms of that certification. All filters, cartridges, and canisters must be labeled with the appropriate NIOSH approval label; the label must not be removed or defaced.

# Medical evaluations

Employees who are required to wear respirators or who choose to wear respirators other than dust masks must have a confidential medical evaluation to ensure that their safety and health is not at risk. Employees are not permitted to wear respirators until a physician has determined that they are medically able to do so. Any employee refusing a medical evaluation will not be permitted to work in areas that require respirators.

A licensed physician at [address of your medical service provider] will conduct medical evaluations.

- The medical evaluation will be conducted with the questionnaire in Appendix C, 1910.134. The program administrator will provide a copy of this questionnaire to each employee who requires a medical evaluation.

- The company will attempt to assist employees who are unable to read the questionnaire. When this is not possible, an employee will be sent directly to the physician for medical evaluation.

- All affected employees will be given a copy of the medical questionnaire and a stamped, pre-addressed envelope to complete and return to the physician. Employees will be permitted to fill out the questionnaire on company time.

- Follow-up medical exams will be granted to employees as required by 1910.134 or [name of the licensed physician].

- All employees will have the opportunity to speak to the physician about their medical evaluation.

- The program administrator will provide [name of the licensed physician] with the following information:

- A copy of this respiratory protection program, a copy of 1910.134, and a list of hazardous substances by work area

- The name of each employee who needs a medical evaluation and his or her work area or job title

- The employee’s proposed respirator type and weight

- The length of time the employee is required to wear the respirator

- The employee’s expected physical work load (light, moderate, or heavy), potential temperature and humidity extremes of the work area, and a description of protective clothing the employee must wear

- Any employee required to wear a positive-pressure air-purifying respirator for medical reasons will be provided with a powered air-purifying respirator.

Additional medical evaluations will be provided under the following circumstances:

- The employee reports signs or symptoms related to his or her ability to use a respirator, such as shortness of breath, dizziness, chest pains, or wheezing

- The supervisor informs the program administrator that the employee needs to be re-evaluated

- Information from this program, including observations made during fit testing and program evaluation, indicates a need for reevaluation

- A change occurs in workplace conditions that may result in an increased physiological burden on the employee

Information from medical examinations and questionnaires is confidential and can be shared only between the employee and the physician.

# Fit Testing

Fit testing is required for employees who wear the following types of respirators: [identify the types of respirators]. Employees who voluntarily wear respirators may also be fit tested upon request.

Employees will be fit tested with the make, model, and size of respirator that they will actually wear. Employees will be provided with several models and sizes of respirators so that they may find an optimal fit. Fit testing of powered air-purifying respirators is to be conducted in the negative pressure mode.

Fit testing will be conducted with one of the acceptable methods shown in the table below:

| Acceptable fit-test methods for typical respirator facepieces |  |  |
| --- | --- | --- |
| Respirator/facepiece | ### Qualitative fit test | Quantitative fit test |
| Half-face negative-pressure air-purifying respirator (including dust masks) | Yes | Yes |
| Full-face negative-pressure air-purifying respirator used in atmospheres up to 10 times the PEL | Yes | Yes |
| Full-face negative-pressure air-purifying respirator used in atmospheres greater than 10 times the PEL | No | Yes |
| Powered air-purifying respirators (PAPRs) | Yes | Yes |
| Supplied-air respirators (SARs) or self-contained breathing apparatus (SCBA) used in the negative-pressure (demand) mode | No | Yes |
| SARs or SCBAs used in the positive-pressure (pressure demand) mode | Yes | Yes |
| SCBAs used for structural fire fighting (positive pressure) | Yes | Yes |
| SCBAs and SARs for atmospheres immediately dangerous to life and health (IDLH), positive pressure | Yes | Yes |

- Qualitative fit tests (QLFT) cannot be used for negative pressure APRs in atmospheres greater than 10 times the PEL.

- Quantitative fit tests (QNFT) must achieve a fit factor of at least 100 for a tight-fitting half mask and at least 500 for a tight-fitting full facepiece.

- Follow the fit-test procedures in 1910.134, Appendix A, regardless of the test method.

# Respirator use

## General use

Employees will use their respirators as required by this program and in accordance with the training they receive. Respirators will not be used in a manner for which NIOSH or the manufacturer does not certify them.

Employees must conduct user seal checks each time that they wear their respirators. Employees must use the positive and negative pressure checks specified in Appendix B-1, 1910.134.

Employees will be permitted to leave their work areas to clean their respirators, to change filters or cartridges, replace parts, or to inspect respirators if they stop functioning. Employees should notify a supervisor before leaving a work area.

Employees are not permitted to wear tight-fitting respirators if they have conditions such as facial scars, facial hair, or missing dentures that prevent them from achieving a good seal. Facial hair must not contact sealing surfaces or interfere with the valve function. Employees are not permitted to wear headphones, jewelry, or other articles that may interfere with the facepiece-to-face seal.

## Voluntary use

Employees who choose to wear respirators will receive a copy of Appendix D, 1910.134, which explains the requirements for voluntary use of respirators. Any employee who voluntarily wears a respirator other than a dust mask is subject to the medical evaluation, cleaning, maintenance, and storage elements of this program, and must be provided with the information specified in this section of the program.

Dust masks: Employees who voluntarily wear dust masks are not subject to the medical evaluation, cleaning, storage, and maintenance provisions of this program.

The program administrator will authorize voluntary use of respirators as requested by all other employees on a case-by-case basis, depending on workplace conditions and medical evaluation results.

## Escape respirator

Escape respirators are for emergencies only and must be inspected before being carried into the workplace. When the emergency alarm sounds, employees in [identify work areas] must immediately don emergency escape respirators, shut down equipment, and evacuate to designated safe areas. The [name of your company] emergency action plan describes emergency evacuation procedures.

## Respirator malfunction

Respirators that are defective or that have defective parts must be removed from service immediately. An employee who discovers a defect in a respirator must inform his or her supervisor who will give the respirators to the program administrator. The program administrator will decide whether to take the respirator out of service, fix it on the spot, or dispose of it.

The employee must discontinue use and inform his or her supervisor that the respirator is not working correctly. The supervisor must ensure that the employee receives parts to repair the respirator or receives another respirator.

## Locations immediately dangerous to life or health

The program administrator has identified the following locations as immediately dangerous to life or health (IDLH): [identify IDLH locations]

- At least one person must stay immediately outside the IDLH atmosphere to respond to emergencies.

- The person entering the IDLH atmosphere and the person outside the IDLH atmosphere must maintain visual, voice, or signal contact.

- The person outside the IDLH atmosphere must be trained and equipped to respond to the emergency.

- The person outside the IDLH atmosphere must use a positive- pressure SCBA or positive-pressure supplied-air respirator with auxiliary SCBA and appropriate rescue retrieval equipment.

- The program administrator or another designated person must be notified before an emergency responder enters the IDLH atmosphere.

# Air quality in atmosphere-supplying respirators

Only Grade D breathing air will be used for atmosphere-supplying respirators. The program administrator will coordinate deliveries of compressed air with the vendor. The vendor must certify that the air in the cylinders meets Grade D breathing-air specifications.

# Cleaning, maintenance, change schedules and storage

## Cleaning

Respirators must be regularly cleaned and disinfected at the respirator cleaning station [identify the location]. Respirators must be cleaned as often as necessary to keep them sanitary.

Atmosphere supplying and emergency use respirators must be cleaned and disinfected after each use.

The following procedure must be used for cleaning and disinfecting respirators:

- Disassemble respirator. Remove filters, canisters, or cartridges.

- Wash the facepiece and parts in warm water with a mild detergent. Do not use organic solvents.

- Rinse completely in clean warm water.

- Wipe the respirator with disinfectant wipes.

- Air dry the respirator in a clean area.

- Reassemble the respirator, inspect it, and replace defective parts.

- Put the respirator in a clean, dry, plastic bag or other air-tight container.

The program administrator will ensure an adequate supply of appropriate cleaning and disinfection materials at the cleaning station. Employees should contact their supervisor or the program administrator when supplies are low.

## Maintenance

Respirators must be properly maintained to ensure that they work properly. Maintenance involves a thorough visual inspection for cleanliness and defects. Worn or deteriorated parts must be replaced. No components will be replaced or repairs made except those recommended by the manufacturer. The manufacturer must repair the regulators or alarms of atmosphere-supplying respirators.

The following checklist must be used to ensure respirators are properly inspected:

| Respirator inspection checklist |  |
| --- | --- |
| Facepiece | No cracks, tears, or holes No facemask distortion No cracked or loose lenses or face shields |
| Head straps | No breaks or tears No broken buckles |
| Valves | No residue or dirt, cracks, or tears in valve material |
| Filters and cartridges | NIOSH approved Gaskets seat properly No cracks or dents in housing Proper cartridge for hazards |
| Air supply systems | Breathing-quality air is used; breathing air meets requirements in ORS 1910.134(i)(5)-(7). Supply hoses are in good condition Hoses are properly connected Settings on regulators and valves are correct |

Employees are permitted to leave their work to maintain their respirators in a designated safe area under the following circumstances:

- To wash their own faces and the respirators’ facepieces to prevent eye or skin irritation

- To replace filters, cartridges, or canisters

- When they detect vapor or gas breakthrough or leakage in the facepiece or detect other damage to the respirator or its components

## Change Schedules

Air-purifying particulate filters, cartridges, or filtering facepieces must be replaced when breathing resistance increases, the cartridge surface is contaminated, or when the filter is damaged.

Note to users of this program. Gases and vapors, odor and irritation are not considered adequate warnings. Because end-of-service-life indicators (ESLI) are available for a limited number of chemicals, employers must develop change-out schedules for cartridges and canisters used with air-purifying respirators for protection against gases and vapors.

## Respirator storage

Respirators must be stored in a clean, dry area in accordance with the manufacturer’s recommendations. Employees must clean and inspect their air-purifying respirators in accordance with the provisions of this program and store them in a plastic bags in their own lockers. Each employee’s name must be on the bag and the bag must be used only to store the respirator.

The program administrator will store unused respirators and respirator components in their original manufacturer’s packaging in [identify location].

# Training

The program administrator will ensure training is provided to respirator users and supervisors. Employees must be trained before using a respirator. Supervisors must be trained before using a respirator or supervising employees who wear respirators. Training will cover the following topics:

- This respiratory protection program

- The Oregon OSHA respiratory protection standard, 1910.134

- Respiratory hazards and their health effects

- Selection and use of respirators

- Limitations of respirators

- How to put on respirators and perform user seal checks

- Fit testing

- Emergency procedures

- Maintenance and storage

- Medical signs and symptoms that limit the use of respirators

Employees must be retrained annually and whenever they change jobs or use a different respirator. Employees must demonstrate their comprehension through hands-on exercises and a written test. The program administrator will document the training, including the type, model, and size of respirator for which each employee has been trained and fit tested.

# Program evaluation

The program administrator will conduct periodic evaluations of the workplace to ensure that the provisions of this program are implemented. Evaluations will include regular consultations with employees who use respirators and their supervisors, site inspections, air monitoring, and a records review. Problems discovered during evaluations must be documented in an inspection log, addressed by the program administrator, and reported to management. The report must recommend how and when to correct each problem.

# Documentation and recordkeeping

A written copy of this program and 1910.134 are kept in the program administrator’s office and are available to employees who wish to review them. Also maintained in the program administrator’s office are copies of training and fit-test records which are updated when new employees are trained, when employees receive refresher training, and when new fit tests are conducted.

Medical questionnaires and the physician’s documented findings are confidential and will remain at [address of your medical service provider]. The program administrator will keep only the physician’s written recommendation for each employee to wear a respirator.

# Appendix D to §1910.134 – Information for employees using respirators when not required under the standard (mandatory)

Respirators are an effective method of protection against designated hazards when properly selected and worn. Respirator use is encouraged, even when exposures are below the exposure limit, to provide an additional level of comfort and protection for workers. However, if a respirator is used improperly or not kept clean, the respirator itself can become a hazard to the worker. Sometimes, workers may wear respirators to avoid exposures to hazards, even if the amount of hazardous substance does not exceed the limits set by OSHA standards. If your employer provides respirators for your voluntary use, of if you provide your own respirator, you need to take certain precautions to be sure that the respirator itself does not present a hazard.

You should do the following:

- Read and heed all instructions provided by the manufacturer on use, maintenance, cleaning and care, and warnings regarding the respirators limitations.

- Choose respirators certified for use to protect against the contaminant of concern. NIOSH, the National Institute for Occupational Safety and Health of the U.S. Department of Health and Human Services, certifies respirators. A label or statement of certification should appear on the respirator or respirator packaging. It will tell you what the respirator is designed for and how much it will protect you.

- Do not wear your respirator into atmospheres containing contaminants for which your respirator is not designed to protect against. For example, a respirator designed to filter dust particles will not protect you against gases, vapors, or very small solid particles of fumes or smoke.

- Keep track of your respirator so that you do not mistakenly use someone else’s respirator.

[63 FR 1270, Jan. 8, 1998; 63 FR 20098, 20099, Apr. 23, 1998]

Stat. Auth.: ORS 654.025(2) and 656.726(3).
Stats. Implemented: ORS 654.001 through 654.295.
Hist: OR-OSHA Admin. Order 3-1998, f. 7/7/98, ef. 7/7/98.

# Fill-in forms

## Form 1: Required respirators — determined through hazard assessments

Note: the first row below is an example.

| ## Required respirators at [name of your company] |  |  |  |
| --- | --- | --- | --- |
| ### Respirator | Area affected | Employees affected | Hazard |
| Filtering facepiece-N95 | Pre-sanding | ##### J. Joplin ##### G. Parsons | ##### Ventilation controls on sanders are in place, but employees continue to be exposed to wood dust levels from 7.0-10.3 mg/m3 (8-hour time-weighted average (TWA). OR-OSHA PEL, 10 mg/m3. |

## Form 2: Respirators for voluntary use — determined through hazard assessments

Note: the first row below is an example.

| ## Respirators for voluntary use at [name of your company] |  |  |  |
| --- | --- | --- | --- |
| ### Respirator | Area affected | Employees affected | Hazard |
| Filtering facepiece-N95 | Assembly | J. Morrison ##### J. Jett | ##### Ventilation controls on sanders are in place. Employee exposures are less that 2.5 mg/m3 (8-hour time-weighted average (TWA). OR-OSHA PEL, 10 mg/m3. Respirators are not required, but dust masks are available for employee use. |

## Form 3: Work areas that require escape respirators

Note: the first row below is an example.

| Employees who work in the following areas must use respirators during emergencies. |  |  |
| --- | --- | --- |
| ## Area | Emergency | Location of escape respirator |
| Spray-booth cleaning area | ##### Spill of hazardous waste | ##### Locker 1 in spray booth area |

## Form 4: Locations immediately dangerous to life or health

The program administrator has identified the following locations as immediately dangerous to life or health (IDLH):

Note: the first row below is an example.

| #### Location | Condition/hazards | Permit-required confined space? | Required respirators |
| --- | --- | --- | --- |
| Dip tank #1 | Employees must periodically enter dip tank #1 for maintenance or repairs. Dip tank #1 exposes employees to orgnic vapors | Yes (Note to users of this program: If “Yes,” employees must also follow requirements in 1910.146, Permit-required confined spaces.) | Full-face pressure demand SCBA |

## Form 5: Medical release

Information provided to the physician

Employee name:

Date:

Job:

Work location:

Type and weight of respirator:

To be used under the following conditions:

- Duration and frequency of use:

- Expected physical effort:

- Additional protective clothing and equipment:

- Environmental temperature and humidity extremes:

Estimated frequency of cartridge/filter replacement:

Medical evaluation: physician release

Is employee medically able to use the respirator?  Yes  No

Identify any limitations on respirator use:

If a follow-up medical evaluation is required, date:

Employee has been given a copy of this recommendation. Yes  No

Signature of physician 
or other licensed health-care provider:

Date:

## Form 6: Medical evaluation questionnaire from 1910.134, Appendix C

To the employer: Answers to questions in Section 1, and to question 9 in Section 2 of Part A, do not require a medical examination.

To the employee: Your employer must allow you to answer this questionnaire during normal working hours or at a time and place that is convenient for you. To maintain your confidentiality, your employer or supervisor must not see your answers. Your employer will tell you how to deliver this questionnaire to the health-care professional who will review it.

To obtain this form in Spanish, go to the CD’s main page and see “Spanish-language resources.”

Part A. Section 1. Mandatory

The following information must be provided by every employee who has been selected to use any type of respirator. (Please print.)

1. 	Today’s date:

2. 	Your name:

3. 	Your age (to nearest year):

4. 	Sex (check one): 	 Male 	 Female

5. 	Your height: 	      feet 	      inches

6. 	Your weight: 	      pounds

7. 	Your job title:

8. 	A phone number where you can be reached by the health-care professional who reviews this questionnaire (Include area code.):

9. 	The best time to phone you at this number:

10. 	Has your employer told you how to contact the health-care professional who will review this questionnaire? (Check one.)    Yes    No

11. 	Check the type of respirator you will use (you can check more than one category):

N, R, or P disposable respirator (filter-mask, non-cartridge type only).

Other type (for example, half- or full-facepiece type, powered air-purifying, supplied-air, self-contained breathing apparatus).

12. Have you worn a respirator? (Check one.)    Yes    No

If yes, what type(s):

Part A. Section 2. Mandatory

Questions 1 through 9 below must be answered by every employee who has been selected to use any type of respirator. (Check “yes” or “no.”)

1.	Do you currently smoke tobacco, or have you smoked 
tobacco in the last month? 	 Yes    No

2. Have you ever had any of the following conditions?

a. Seizures (fits): 	 Yes    No

b. Diabetes (sugar disease) 	 Yes    No

c. Allergic reactions that interfere with your breathing 	 Yes    No

d. Claustrophobia (fear of closed-in places) 	 Yes    No

e. Trouble smelling odors 	 Yes    No

3. 	Have you ever had any of the following pulmonary or lung problems?

a. Asbestosis 	 Yes    No

b. Asthma 	 Yes    No

c. Chronic bronchitis	 Yes    No

d. Emphysema	 Yes    No

e. Pneumonia	 Yes    No

f. Tuberculosis	 Yes    No

g. Silicosis	 Yes    No

h. Pneumothorax (collapsed lung) 	 Yes    No

i. Lung cancer	 Yes    No

j. Broken ribs	 Yes    No

k. Chest injuries or chest surgeries	 Yes    No

l. Any other lung problem that you’ve been told about	 Yes    No

4. 	Do you currently have any of the following symptoms of pulmonary or lung illness?

a.	Shortness of breath	 Yes    No

b. 	Shortness of breath when walking fast on level ground or walking up 
a slight hill or incline	 Yes    No

c. 	Shortness of breath when walking with other people at an ordinary 
pace on level ground	 Yes    No

d.	Do you have to stop for breath when walking at your own 
pace on level ground	 Yes    No

e. 	Do you have shortness of breath when washing or dressing yourself	 Yes    No

f. 	Do you have shortness of breath that interferes with your job	 Yes    No

g. 	Do you have coughing that produces phlegm (thick sputum) 	 Yes    No

h. 	Do you have coughing that wakes you early in the morning	 Yes    No

i. 	Do you have coughing that occurs mostly when you are lying down	 Yes    No

j.	Have you coughed up blood in the last month	 Yes    No

k.	Do you wheeze	 Yes    No

l.	Do you have wheezing that interferes with your job	 Yes    No

m.	Do you have chest pain when you breathe deeply	 Yes    No

n.	Do you have any other symptoms that you think 
may be related to lung problems	 Yes    No

5. 	Have you ever had any of the following cardiovascular or heart problems?

a. Heart attack	 Yes    No

b. Stroke	 Yes    No

c. Angina	 Yes    No

d. Heart failure	 Yes    No

e. Swelling in your legs or feet (not caused by walking)	 Yes    No

f. Heart arrhythmia (heart beating irregularly)	 Yes    No

g. High blood pressure	 Yes    No

h. Any other heart problem that you’ve been told about	 Yes    No

6. 	Have you ever had any of the following cardiovascular or heart symptoms?

a. Frequent pain or tightness in your chest	 Yes    No

b. Pain or tightness in your chest during physical activity	 Yes    No

c. Pain or tightness in your chest that interferes with your job	 Yes    No

d. In the past two years, have you noticed your heart 
skipping or missing a beat	 Yes    No

e. Heartburn or indigestion that is not related to eating	 Yes    No

f. Any other symptoms that you think may be related to 
heart or circulation problems	 Yes    No

7. 	Do you take medication for any of the following problems?

a. Breathing or lung problems	 Yes    No

b. Heart trouble	 Yes    No

c. Blood pressure	 Yes    No

d. Seizures (fits) 	 Yes    No

8. 	If you’ve used a respirator, have you ever had any of the following problems? 
(If you’ve never used a respirator, go to question 9.)

a. Eye irritation	 Yes    No

b. Skin allergies or rashes	 Yes    No

c. Anxiety	 Yes    No

d. General weakness or fatigue	 Yes    No

e. Any other problem that interferes with your use of a respirator	 Yes    No

9.	Would you like to talk to the health care professional who will 
review this questionnaire about your answers to this questionnaire	 Yes    No

Questions 10 to 15 below must be answered by every employee who has been selected to use either a full-facepiece respirator or a self-contained breathing apparatus (SCBA). For employees who have been selected to use other types of respirators, answering these questions is voluntary.

10. 	Have you ever lost vision in either eye (temporarily or permanently)	 Yes    No

11. 	Do you have any of the following vision problems?

a. Wear contact lenses	 Yes    No

b. Wear glasses	 Yes    No

c. Color blind	 Yes    No

d. Any other eye or vision problem	 Yes    No

12. 	Have you ever had an injury to your ears, including a broken ear drum	 Yes    No

13. 	Do you currently have any of the following hearing problems?

a. Difficulty hearing	 Yes    No

b. Wear a hearing aid	 Yes    No

c. Any other hearing or ear problem	 Yes    No

14. 	Have you ever had a back injury	 Yes    No

15. 	Do you currently have any of the following musculoskeletal problems?

a. Weakness in your arms, hands, legs, or feet	 Yes    No

b. Back pain	 Yes    No

c. Difficulty moving your arms and legs	 Yes    No

d. Pain or stiffness when you lean forward or backward at the waist	 Yes    No

e. Difficulty fully moving your head up or down	 Yes    No

f. Difficulty fully moving your head side to side 	 Yes    No

g. Difficulty bending at your knees	 Yes    No

h. Difficulty squatting to the ground	 Yes    No

i. Climbing a flight of stairs or a ladder carrying more than 25 pounds	 Yes    No

j. Any other muscle or skeletal problem that interferes 
with using a respirator	 Yes    No

Part B. Section 2

Any of the following questions as well as questions not listed here may be added to the questionnaire at the discretion of the health-care professional who will review the questionnaire.

1. 	In your present job, are you working at high altitudes (over 5,000 feet) 
or in a place that has lower than normal amounts of oxygen?	 Yes    No

If yes, do you have feelings of dizziness, shortness of breath, 
pounding in your chest, or other symptoms when you’re working 
under these conditions?	 Yes    No

2. 	At work or at home, have you ever been exposed to hazardous solvents, 
hazardous airborne chemicals (e.g., gases, fumes, or dust), or have you 
come into skin contact with hazardous chemicals? 	 Yes    No

If yes, name the chemicals, if you know them:

3. 	Have you ever worked with any of the materials or under any of the conditions listed below:

a. Asbestos	 Yes    No

b. Silica (e.g., in sandblasting) 	 Yes    No

c. Tungsten/cobalt (e.g., grinding or welding this material) 	 Yes    No

d. Beryllium	 Yes    No

e. Aluminum	 Yes    No

f. Coal (for example, mining) 	 Yes    No

g. Iron	 Yes    No

h. Tin	 Yes    No

i. Dusty environments	 Yes    No

j. Any other hazardous exposures	 Yes    No

If yes, describe these exposures:

4. 	List any second jobs or side businesses you have:

5. 	List your previous occupations:

6. 	List your current and previous hobbies:

7. 	Have you been in the military services? 	 Yes    No

If yes, were you exposed to biological or chemical agents 
(either in training or combat)? 	 Yes    No

8. 	Have you ever worked on a HAZMAT team? 	 Yes    No

9. 	Other than medications for breathing and lung problems, heart trouble, 
blood pressure, and seizures mentioned earlier in this questionnaire, 
are you taking any other medications for any reason (including 
over-the-counter medications)? 	 Yes    No

If yes, name the medications, if you know them:

10. 	Will you be using any of the following items with your respirator(s)?

a. HEPA filters	 Yes    No

b. Canisters (for example, gas masks) 	 Yes    No

c. Cartridges	 Yes    No

11. 	How often are you expected to use the respirator(s)? Check yes or no for all answers that apply to you.

a. Escape only (no rescue) 	 Yes    No

b. Emergency rescue only	 Yes    No

c. Less than 5 hours per week	 Yes    No

d. Less than 2 hours per day	 Yes    No

e. 2 to 4 hours per day	 Yes    No

f. Over 4 hours per day	 Yes    No

12. 	During the period you are using the respirator(s), is your work effort:

a. Light	 Yes    No

If yes, how long does this period last during the average shift? hours:       minutes:

Examples of a light work effort are sitting while writing, typing, drafting, or performing light assembly work; standing while operating a drill press (1-3 lbs.) controlling machines.

b. Moderate	 Yes    No

If yes, how long does this period last during the average shift? hours:       minutes:

Examples of moderate work effort are sitting while nailing or filing; driving a truck or bus in urban traffic; standing while drilling, nailing, performing assembly work, or transferring a moderate load (about 35 lbs.) at trunk level; walking on a level surface about 2 mph or down a 5-degree grade about 3 mph; or pushing a wheelbarrow with a heavy load (about 100 lbs.) on a level surface.

c. Heavy	 Yes    No

If yes, how long does this period last during the average shift; hours:       minutes:

Examples of heavy work are lifting a heavy load (about 50 lbs.) from the floor to your waist or shoulder; working on a loading dock; shoveling; standing while bricklaying or chipping castings; walking up an 8-degree grade about 2 mph; climbing stairs with a heavy load (about 50 lbs.).

13. 	Will you be wearing protective clothing and/or equipment 
(other than the respirator) when you’re using your respirator? 	 Yes    No

If yes, describe this protective clothing and/or equipment:

14. 	Will you be working under hot conditions? 
(temperature exceeding 77°F) 	 Yes    No

15. 	Will you be working under humid conditions? 	 Yes    No

16. 	Describe the work you’ll be doing while you’re using your respirator(s):

17. 	Describe any special or hazardous conditions you might encounter when you’re using your respirator(s) (for example, confined spaces, life-threatening gases):

18. 	Provide the following information, if you know it, for each toxic substance that you’ll be exposed to when you’re using your respirator(s):

• Name of the first toxic substance:

• Estimated maximum exposure level per shift:

• Duration of exposure per shift:

• Name of the second toxic substance:

• Estimated maximum exposure level per shift:

• Duration of exposure per shift:

• Name of the third toxic substance:

• Estimated maximum exposure level per shift:

• Duration of exposure per shift:

• Name of any other toxic substances you’ll be exposed to while using your respirator:

19. 	Describe any special responsibilities you’ll have while using your respirator(s) that may affect the safety and well being of others (for example, rescue, or security):

## Form 7: Fit test results

Note: Appendix A, 1910.134, “Fit Testing Procedures,” is required for fit testing.

Date:

Employee name:

Job:

Department:

Fit test method:

| Type of respirator | Make/model/size | Fit factor/results |
| --- | --- | --- |

Person performing the fit test:

Problems the employee has encountered with his/her respirators:

## Form 8: Program evaluation summary

Evaluator:

Date:

Names of employees consulted during the evaluation:

| Problems identified | Corrective action |
| --- | --- |